AEGISAI

AI-ENABLED REAL-TIME DIGITAL TWIN SYSTEM FOR AERO-PISTON ENGINE HEALTH MONITORING, FAULT PREDICTION & MISSION RELIABILITY

1. EXECUTIVE SUMMARY

AegisAI is a real-time AI and Digital Twin platform designed to monitor the health of an aero-piston engine used in a MALE UAV environment.

The system continuously receives engine telemetry such as RPM, temperature, pressure, vibration, oil condition and other available engine-health parameters. The incoming data is filtered, synchronized and transformed into meaningful health features. An AI/ML layer then identifies abnormal behaviour, estimates engine health, detects emerging fault patterns and predicts degradation.

At the same time, a Digital Twin maintains a virtual representation of the engine. Instead of simply displaying sensor values, the twin compares:

What the engine should be doing → What the engine is actually doing → How the difference is changing over time

This enables AegisAI to move from reactive monitoring toward predictive health management.

Core workflow:

Physical Engine → Sensors → Data Acquisition → Edge Processing → AI/ML → Digital Twin → Health Index → Fault Prediction → Mission-Reliability Analysis → Operator Dashboard

2. PROBLEM STATEMENT

MALE UAVs are required to operate for long durations, making propulsion-system reliability particularly important.

Traditional monitoring approaches commonly depend heavily on fixed thresholds. A parameter may be flagged when it exceeds a predefined limit, but this does not necessarily answer:

• Is the engine gradually degrading?

• Is the abnormality actually caused by a sensor?

• Is the engine behaviour inconsistent with its expected operating state?

• How quickly is the condition worsening?

• What could happen if the current trend continues?

• Is the engine healthy enough for the planned mission profile?

AegisAI aims to transform raw telemetry into:

Data → Information → Health → Prediction → Decision Support

3. PROPOSED SOLUTION

AegisAI creates a continuously updated virtual representation of the physical engine.

The system consists of five major intelligence layers:

Layer 1 — Physical Monitoring

Collect engine parameters through sensors or available ECU/DAQ telemetry.

Layer 2 — Data Intelligence

Clean, synchronize and extract useful features from raw signals.

Layer 3 — AI Health Intelligence

Detect anomalies, classify faults where validated training data exists and calculate an Engine Health Index.

Layer 4 — Digital Twin

Compare real engine behaviour with expected behaviour and continuously update the virtual engine state.

Layer 5 — Mission Decision Support

Translate engine-health information into understandable risk, trend and maintenance indicators.

4. CORE ARCHITECTURE

PHYSICAL ENGINE

Aero-Piston Engine

↓

SENSOR / DAQ LAYER

RPM | Temperature | Pressure | Vibration | Oil | Other Parameters

↓

EDGE DATA PROCESSING

Filtering | Synchronization | Feature Extraction

↓

AI / ML ENGINE

Anomaly Detection | Fault Classification | Health Prediction | Engine Health Index

↓

DIGITAL TWIN CORE

Live State | Expected Behaviour | Degradation Modelling | Simulation | What-If Analysis

↓

HEALTH & RISK

Health Status | Anomaly Score | Trend | Confidence | Risk

↓

OPERATOR DASHBOARD

Real-Time Monitoring | Alerts | Digital Twin | AI Predictions | Mission Insights

5. SENSOR & DATA ACQUISITION LAYER

Potential engine-health parameters include:

• RPM — Engine operating state

• Cylinder/engine temperature — Thermal health

• Exhaust temperature — Combustion/performance indication

• Oil temperature — Lubrication and thermal condition

• Oil pressure — Lubrication-system health

• Fuel flow — Fuel-system/performance behaviour

• Vibration — Mechanical abnormality

• Pressure — Operating-condition assessment

• Electrical/alternator data — Auxiliary-system health

• Throttle/load — Operating context

The exact sensor set should be finalized according to the selected engine and test setup.

6. DATA PROCESSING

Raw sensor data should not directly enter the AI model.

AegisAI performs:

Signal Filtering

Removes noise and unrealistic spikes.

Synchronization

Aligns parameters arriving at different sampling rates.

Missing-Data Handling

Identifies missing or corrupted telemetry.

Feature Extraction

• RPM variation

• Temperature trend

• Rate of temperature rise

• Pressure deviation

• Vibration RMS

• Vibration frequency features

• Oil-temperature trend

• Temperature/RPM relationship

• Pressure/RPM relationship

• Sensor correlation

This converts:

Raw Telemetry → Engine Behaviour Features

7. EXPECTED VS ACTUAL ENGINE BEHAVIOUR

One of the strongest technical ideas in AegisAI is residual-based health monitoring.

Instead of asking only:

“Is temperature above the threshold?”

AegisAI asks:

“Given the current RPM, load and operating conditions, what temperature should the engine produce?”

Residual:

Residual = Actual Value − Expected Value

Example:

Expected temperature = 165°C

Actual temperature = 179°C

Residual = +14°C

A single residual does not necessarily mean failure.

However, if the system observes:

+3°C → +6°C → +10°C → +14°C → +19°C

the increasing trend becomes meaningful.

This allows AegisAI to detect deviation from expected engine behaviour rather than relying only on fixed threshold violations.

8. HYBRID DIGITAL TWIN

AegisAI should preferably use a Hybrid Digital Twin rather than claiming a purely physics-based or purely AI-based twin.

Physics/Engineering Model:

Represents expected engine behaviour.

Sensor Data:

Represents the real physical engine.

AI/ML:

Learns patterns that are difficult to capture through simplified models alone.

Digital Twin:

Combines these representations.

REAL ENGINE

↓

Sensor Data

↓

Digital Twin

├── Physics Model

├── Sensor Data

└── AI/ML

↓

Estimated Engine State

9. AI/ML LAYER

AegisAI should use different models for different tasks rather than relying on one AI model for everything.

A. Anomaly Detection

Detects behaviour that differs significantly from learned healthy behaviour.

Possible approaches:

• Isolation Forest

• Autoencoder

• One-Class SVM

• Statistical residual analysis

Output:

Normal / Anomalous

B. Fault Classification

If sufficient labelled fault data becomes available, the system can classify probable fault categories.

Example:

Anomaly Detected

↓

Feature Analysis

↓

Fault Classifier

↓

Possible Fault:

Lubrication / Combustion / Sensor / Thermal / Mechanical

For the prototype, simulated or controlled fault data should be clearly labelled as simulation/validation data rather than real operational DRDO data.

10. ENGINE HEALTH INDEX

AegisAI can combine multiple health indicators into a single understandable value.

Engine Health Index can combine:

• Thermal health

• Lubrication health

• Vibration health

• RPM stability

• Pressure health

• Anomaly score

• Degradation trend

Example dashboard:

ENGINE HEALTH

92

GOOD

Thermal: NORMAL

Vibration: NORMAL

Oil Pressure: NORMAL

RPM: NORMAL

Trend: STABLE

The exact weights should be experimentally validated rather than presented as authoritative values without testing.

11. CONFIDENCE-AWARE AI

AegisAI should show confidence along with predictions.

Instead of:

“Fault probability = 87%”

show:

“Possible thermal abnormality”

Confidence: 87%

Evidence:

• Rising temperature residual

• Abnormal RPM relationship

This prevents the dashboard from presenting uncertain predictions as absolute facts.

12. SENSOR-FAULT AWARENESS

AegisAI should distinguish:

Engine abnormality

from

Sensor abnormality.

Example:

Temperature Sensor

↓

Sudden unrealistic jump

↓

Other sensors remain normal

↓

Cross-sensor validation

↓

Possible SENSOR FAULT

Instead of immediately reporting:

“ENGINE FAILURE”

the system can identify that the abnormal signal may originate from the sensor itself.

This improves reliability and reduces unnecessary false alarms.

13. DIGITAL TWIN STATE

The Digital Twin should display:

Live State

Current operating condition.

Expected State

Model-predicted healthy condition.

Difference

Deviation between actual and expected behaviour.

Degradation

Long-term change in engine behaviour.

Prediction

Potential future condition.

Simulation

Behaviour under selected operating conditions.

14. MISSION-AWARE HEALTH MONITORING

AegisAI can associate engine health with the mission profile.

Example:

MISSION PROFILE

↓

Take-Off

↓

Climb

↓

Cruise

↓

High-Endurance Operation

↓

Return

The system can evaluate:

• Engine condition during each phase

• Abnormal trends during high-load periods

• Predicted degradation

• Health margin

• Mission-specific risk indicators

This transforms:

Engine Monitoring

into:

Engine + Mission Context Monitoring

15. MISSION REPLAY

AegisAI stores historical telemetry.

Workflow:

Select Mission

↓

Replay Timeline

↓

Engine Parameters

↓

Digital Twin State

↓

AI Predictions

↓

Anomalies

↓

Health Trend

The operator can move through the timeline and identify when the Digital Twin first detected a deviation.

Mission replay supports:

• Post-flight analysis

• Maintenance investigation

• Model validation

• Training

• Identification of recurring patterns

16. WHAT-IF SIMULATION

The Digital Twin can support controlled simulation scenarios.

Example:

Scenario A — Normal Cruise

Scenario B — Increased Thermal Load

Scenario C — Developing Lubrication Abnormality

Scenario D — Rapid Throttle Transition

The system compares:

Expected Behaviour

VS

Simulated Behaviour

VS

Observed Behaviour

This makes the Digital Twin more than a visualization tool.

17. PREDICTIVE MAINTENANCE

AegisAI moves from:

“Something is wrong.”

to:

“Something is gradually changing.”

and eventually:

“Maintenance attention may be required if this trend continues.”

The system can generate:

LOW → MEDIUM → HIGH → CRITICAL

Maintenance priority based on:

• Anomaly severity

• Trend

• Persistence

• Confidence

• Health Index

• Operating contex

18. REMAINING USEFUL LIFE (RUL)

RUL should not be claimed as a reliable numerical prediction unless suitable degradation/run-to-failure data and a defensible end-of-life definition are available.

For the prototype, AegisAI can implement:

Degradation Trend Estimation

and later extend it to:

Remaining Useful Life Prediction

Reliable RUL requires sufficient historical degradation data and proper validation.

19. EXPLAINABLE AI

AegisAI should answer:

“Why did the system generate this alert?”

Example:

ALERT:

Thermal Health Warning

Confidence: 89%

Main Contributing Factors:

1. Increasing exhaust-temperature trend

2. Temperature/RPM deviation

3. Abnormal pressure relationship

This is more useful than simply showing:

“FAULT DETECTED”

The operator can understand the evidence behind the prediction.

20. OPERATOR DASHBOARD

Recommended dashboard layout:

AEGISAI

REAL-TIME ENGINE HEALTH MONITOR

ENGINE HEALTH

92

GOOD

DIGITAL TWIN

Live Engine Model

ALERTS

Normal

No Critical Alerts

RPM

XXXX

TEMPERATURE

XXX °C

OIL PRESSURE

XX bar

AI PREDICTION / TREND

MISSION STATUS | RISK | RECOMMENDATION | REPLAY

21. AEGISAI ALERT LEVELS

GREEN — NORMAL

Engine behaviour matches expected behaviour.

BLUE — OBSERVATION

Minor deviation detected.

AMBER — WARNING

Persistent abnormal trend detected.

RED — CRITICAL

High-confidence severe abnormality requiring immediate engineering/operator attention.

These are decision-support states and should not be treated as autonomous engine-control commands unless independently validated and approved for that purpose.

22. WHAT MAKES AEGISAI DIFFERENT

The strongest differentiation is not simply:

“We use AI.”

Instead, AegisAI combines:

1. Real-time telemetry

+

2. Expected engine behaviour

+

3. AI anomaly detection

+

4. Digital Twin synchronization

+

5. Engine Health Index

+

6. Sensor-vs-engine anomaly reasoning

+

7. Mission-context analysis

+

8. Mission replay

+

9. Explainable alerts

+

10. Confidence-aware predictions

into one integrated workflow.

The key contribution is the integration of these capabilities into a unified engine-health decision-support platform.

23. PROPOSED INNOVATIONS

1. Residual-Based Engine Health Intelligence

Actual engine behaviour is continuously compared against expected behaviour.

2. Hybrid Digital Twin

Physics/engineering behaviour + sensor telemetry + AI.

3. Sensor-vs-Engine Fault Separation

Attempts to distinguish bad measurements from genuine engine abnormalities.

4. Mission-Aware Health Score

Engine health is interpreted according to the operating/mission phase.

5. Explainable Alerts

The system shows why an abnormality was detected.

6. Mission Replay

Historical engine behaviour can be reconstructed and analyzed.

7. What-If Digital Twin

Controlled scenarios can be simulated without physically stressing an engine.

8. Confidence-Aware Predictions

Predictions are accompanied by confidence/uncertainty rather than false precision.

24. TECHNICAL STACK

Hardware / Data Layer:

• Sensors

• DAQ / Microcontroller

• ESP32 or suitable controller for prototype

• CAN/Serial interface where available

• Local edge computer

Backend:

• Python

• FastAPI / Flask

• NumPy

• Pandas

• SciPy

Machine Learning:

• Scikit-learn

• PyTorch / TensorFlow where required

Digital Twin:

• Python-based engine model

• Physics/engineering equations

• State estimation

• Real-time synchronization

Database:

• PostgreSQL / SQLite

• Time-series storage where appropriate

Dashboard:

• React

• JavaScript / TypeScript

• Plotly / ECharts / Chart.js

• 3D visualization where required

Deployment:

• Edge computer

• Local server

• Ground monitoring workstation

25. PROTOTYPE DEVELOPMENT STRATEGY

PHASE 1 — DATA LAYER

Build:

Sensor → DAQ → Database

Validate:

• Sampling

• Timestamping

• Noise

• Missing data

PHASE 2 — HEALTH MONITORING

Implement:

• Filtering

• Feature extraction

• Baseline

• Health indicators

PHASE 3 — AI

Implement:

Anomaly Detection → Fault Classification → Health Prediction

PHASE 4 — DIGITAL TWIN

Implement:

Expected Behaviour → Live Synchronization → Deviation Detection

PHASE 5 — MISSION ENGINE

Implement:

Mission Profile → Simulation → Replay → Health Evaluation

PHASE 6 — DASHBOARD

Integrate all modules into one operator interface.

26. DEMONSTRATION SCENARIO

Step 1:

Start the engine simulation in a normal state.

Dashboard shows:

HEALTH = GOOD

Step 2:

Introduce a controlled abnormal condition in the simulator.

Step 3:

Raw sensor values begin changing.

Step 4:

Digital Twin predicts expected behaviour.

Step 5:

Residual begins increasing.

Step 6:

AI identifies an anomaly.

Step 7:

Health Index gradually decreases.

Step 8:

Dashboard explains the contributing parameters.

Step 9:

Mission-risk indicator changes.

Step 10:

Open Mission Replay and show the point where degradation began.

This demonstrates the complete:

Sensor → AI → Digital Twin → Prediction → Decision Support

workflow.

27. VALIDATION STRATEGY

AegisAI should not be validated only on the same data used for training.

Training Data:

Healthy + controlled/simulated degradation data.

Validation Data:

Different operating conditions.

Test Data:

Previously unseen scenarios.

Metrics:

• Anomaly detection precision

• Recall

• F1 score

• False-alarm rate

• Detection latency

• Health-index stability

• Prediction error

• Digital Twin tracking error

Most importantly, compare:

Threshold-Only Monitoring

versus

AegisAI

This provides a measurable demonstration of improvement.

28. DATA LIMITATION

A major challenge is data availability.

Many public aero-engine datasets concern turbofan/gas-turbine engines rather than the specific aero-piston engine class required here.

Therefore, AegisAI should use a combination of:

• Available real sensor data

• Controlled laboratory/test-rig data where available

• Physics-based simulation

• Carefully labelled synthetic fault scenarios

The report and dashboard should clearly identify whether results come from real measurements, simulation or synthetic data.

No simulated result should be presented as real operational DRDO/UAV performance.

29. KEY PERFORMANCE INDICATORS

Recommended KPIs:

• Data latency — Low

• Dashboard refresh — Real-time

• Anomaly detection — High precision/recall

• False alarms — Minimized

• Digital Twin tracking error — Minimized

• Health-index stability — High

• Fault detection latency — Low

• Explainability — Available

• Mission replay — Supported

• Sensor fault discrimination — Supported

Do not use invented percentages in the final report.

Replace target values with measured experimental results after testing.

30. EXPECTED IMPACT

OPERATIONAL IMPACT

• Earlier identification of abnormal engine behaviour

• Improved engine-health visibility

• Reduced unexpected maintenance interruptions

• Better post-test/post-flight analysis

TECHNICAL IMPACT

• Integration of AI + Digital Twin

• Continuous health estimation

• Degradation tracking

• Explainable diagnostics

MAINTENANCE IMPACT

• Condition-based maintenance support

• Trend-based maintenance planning

• Historical engine-health records

SCALABILITY IMPACT

Engine → UAV → Fleet → Ground Monitoring System

The architecture can eventually support fleet-level health analytics without fundamentally changing the core monitoring architecture.

31. LIMITATIONS

1. Limited Real Fault Data

Synthetic/controlled data cannot completely represent real-world failures.

2. Engine-Specific Modelling

Different engines have different thermodynamic and mechanical characteristics.

3. Sensor Uncertainty

Sensor drift and noise can influence AI predictions.

4. RUL Uncertainty

Reliable RUL requires suitable degradation history and an end-of-life definition.

5. Digital Twin Fidelity

The virtual engine must be continuously calibrated against real measurements.

6. Safety

AegisAI should be treated as a decision-support system rather than an autonomous engine-control system unless independently validated and approved for that role.

32. FUTURE DEVELOPMENT

LEVEL 1

Prototype with simulated engine data.

LEVEL 2

Connect to an instrumented laboratory/test engine.

LEVEL 3

Validate the hybrid Digital Twin against real telemetry.

LEVEL 4

Deploy AI processing at the edge.

LEVEL 5

Develop fleet-level analytics.

LEVEL 6

Add secure historical health records.

LEVEL 7

Expand to additional compatible engine configurations.

LEVEL 8

Improve mission-context modelling and predictive maintenance.

LEVEL 9

Integrate advanced uncertainty estimation and explainable AI.

LEVEL 10

Develop a validated real-world engine-health management platform.

33. RESEARCH FOUNDATION

AegisAI is grounded in established research rather than treating Digital Twin as merely a visualization technology.

Key research areas include:

• Aircraft propulsion health monitoring

• Digital Twins for aircraft maintenance and operation

• Dynamic data-driven Digital Twins for aircraft-engine health management

• Digital Twin-based aero-engine health assessment

• AI/deep-learning-based fault diagnosis

• Aircraft powertrain Digital Twin health management

• Data-driven aero-engine degradation and RUL prediction

Important references:

1. NASA Glenn Research Center — Aircraft Propulsion Health Monitoring

https://ntrs.nasa.gov/citations/20120002624

2. Digital Twins for Aircraft Maintenance and Operation

https://doi.org/10.1016/j.iot.2023.100991

3. Dynamic Data-Driven Digital Twin for Aircraft Engine Health Management

https://doi.org/10.1016/j.promfg.2021.10.020

4. Aero-Engine Gas Path Health Assessment Based on Digital Twin

https://doi.org/10.1016/j.engfailanal.2022.106790

5. NASA — Digital Twin for Aircraft Powertrain Health Management

https://ntrs.nasa.gov/citations/20260004592

6. IEEE — Data-Driven Digital Model for Remaining Useful Life Prediction of Aero-Engines

https://ieeexplore.ieee.org/document/10778830

34. FINAL INNOVATION STATEMENT

AegisAI is an integrated, mission-aware engine health intelligence platform that combines real-time telemetry, hybrid Digital Twin modelling, AI-based anomaly detection, health-index estimation, explainable diagnostics, degradation tracking and mission replay.

Its key contribution is not the use of any single AI or Digital Twin technique, but the integration of these capabilities into a continuous physical-to-digital-to-decision workflow tailored to aero-piston engine health management.

The architecture is designed to progress from a simulation-based demonstrator toward validation with real engine-test data, enabling progressively stronger evidence for predictive maintenance and mission-reliability applications.

FINAL DIFFERENTIATION STATEMENT:

Existing research demonstrates individual Digital Twin, AI and aero-engine health-management techniques. AegisAI integrates these capabilities into a mission-aware, real-time architecture focused on aero-piston engine health assessment and decision support.

CORE AEGISAI PIPELINE:

ENGINE

↓

SENSORS

↓

REAL-TIME DATA

↓

EDGE PROCESSING

↓

AI / ML

↓

EXPECTED VS ACTUAL BEHAVIOUR

↓

DIGITAL TWIN

↓

ENGINE HEALTH INDEX

↓

ANOMALY / FAULT PREDICTION

↓

MISSION-AWARE RISK

↓

EXPLAINABLE DECISION SUPPORT

↓

OPERATOR DASHBOARD

↓

MISSION REPLAY & PREDICTIVE MAINTENANCE